ISO #####:####(E)

ISO TC 20/SC 14/WG 1

Date: 2024-03-01

Space systems — Contemporary lunar reference system

Preliminary Work Item

Warning for draft documents

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

© ISO 2024

All rights reserved. Unless otherwise specified, or required in the context of its implementation, no part of this publication may be reproduced or utilized otherwise in any form or by any means, electronic or mechanical, including photocopying, or posting on the internet or an intranet, without prior written permission. Permission can be requested from either ISO at the address below or ISO’s member body in the country of the requester.

ISO copyright office

CP 401 • Ch. de Blandonnet 8

CH-1214 Vernier, Geneva

Phone: +41 22 749 01 11

Email: copyright@iso.org

Website: www.iso.org

Published in Switzerland

Contents

Foreword	iv

Introduction	v

1	Scope	1

2	Normative references	1

3	Terms and definitions	1

4	Abbreviated terms	2

5	Definition	2

5.1	Lunar polar axis	2

5.2	Lunar prime meridian	2

5.3	Lunar reference ellipsoid	2

5.4	Lunar orthogonal frame	2

5.5	Lunar surface reference	3

5.5.1	Latitude	3

5.5.2	Longitude	3

5.6	Lunar vertical reference	3

5.6.1	Altitude	3

5.6.2	Equipotential surface	4

6	Realization	4

Annex A (informative)  Lunar basic characteristics	5

Annex B (informative)  Topographic map example	6

Annex C (informative)  Selenodesy short history	9

Bibliography	10

Foreword

ISO (the International Organization for Standardization) is a worldwide federation of national standards bodies (ISO member bodies). The work of preparing International Standards is normally carried out through ISO technical committees. Each member body interested in a subject for which a technical committee has been established has the right to be represented on that committee. International organizations, governmental and non-governmental, in liaison with ISO, also take part in the work. ISO collaborates closely with the International Electrotechnical Commission (IEC) on all matters of electrotechnical standardization.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of ISO documents should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives).

ISO draws attention to the possibility that the implementation of this document may involve the use of a patents. ISO takes no position concerning the evidence, validity or applicability of any claimed patent rights in respect thereof. As of the date of publication of this document, ISO had not received notice of a patents which may be required to implement this document. However, implementers are cautioned that this may not represent the latest information, which may be obtained from the patent database available at www.iso.org/patents. ISO shall not be held responsible for identifying any or all such patent rights.

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT), see www.iso.org/iso/foreword.html.

This document was prepared by Technical Committee ISO/TC 20, Aircraft and space vehicles, Subcommittee SC 14, Space systems and operations.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html.

Introduction

In the 2020s, we humans again increased our activities related to development for the Moon of the Earth. However, we have a problem: there is no official standard for the coordinate system. To solve this problem, this document describes basic information about the Moon's coordinate reference system and its specification rules.

Some are cautious about creating such a standard. But we need to hurry. Because experts will be working for the Moon and need a coordinate system to ensure their safety and security. Despite the urgency of this request, this document has been carefully crafted to respect the future work of expertized geodesists (plan).

Figure 1 is the near side of the Moon, north at top, as seen from Earth.

The Moon of the Earth

Space systems — Lunar reference system for spatial coordinates

Scope

This document define the lunar reference system for the spatial coordinates of the Moon of the Earth.

Normative references

There are no normative references in this document.

Terms and definitions

For the purposes of this document, the following terms and definitions apply.

ISO and IEC maintain terminology databases for use in standardization at the following addresses:

- ISO Online browsing platform: available at https://www.iso.org/obp

- IEC Electropedia: available at https://www.electropedia.org/

3.1

latitude

angle from the equatorial plane to the perpendicular to the ellipsoid through a given point, northwards treated as positive

[SOURCE: ISO 19111:2019, 3.1.32 ― ‘Geodetic’ and ‘ellipsoidal’ have been omitted.]

3.2

longitude

angle from the prime meridian plane to the meridian plane of a given point, eastward treated as positive

[SOURCE: ISO 19111:2019, 3.1.33 ― ‘Geodetic’ and ‘ellipsoidal’ have been omitted.]

3.3

meridian

intersection of an ellipsoid by a plane containing the shortest axis of the ellipsoid

Note 1 to entry: This term is generally used to describe the pole-to-pole arc rather than the complete closed figure

[SOURCE: ISO 19111:2019, 3.1.42]

3.4

prime meridian

meridian from which the longitudes of other meridians are quantified

[SOURCE: ISO 19111:2019, 3.1.50]

3.5

equatorial plane

plane through a designated center of an object and normal to the rotational axis of the object

[SOURCE: ISO 19026:2009, 3.1.3]

3.6

north pole

pole of rotation that lies on the north side of invariable plane the solar system

[SOURCE: ISO 18026:2009, 3.1.5]

3.7

south pole

pole of rotation that lies on the south side of the invariable plane of the solar system

[SOURCE: ISO 18026:2009]

3.8

altitude

distance of a point from the reference surface positive if upwards or outside of the reference ellipsoid

- Abbreviated terms

DE		Development Ephemeris

LALT	Laser Altimeter

LLR		Lunar Laser Ranging

ME		Mean Earh/polar axis

Definition

Lunar polar axis

The lunar polar axis shall be defined by the axis between the mean rotational poles.

Lunar prime meridian

The lunar prime meridian shall be defined by the mean Earth direction. The prime meridian is zero longitude.

Note to entry: see

Lunar reference ellipsoid

The lunar reference ellipsoid shall be defined by the followings: The semi-major axis is 1 734 400 m and the semi-minor axis is 1 734 400 m, therefore the flattening is 0. The polar axis is the semi-minor axis. The above reference ellipsoid is a true sphere.

Lunar orthogonal frame

The lunar geocentric orthogonal coordinate system is a coordinate system shall consist for three axes; X-axis, Y-axis, and Z-axis, orthogonal to each other at the center of flat rotating ellipsoid as Figure 1. Requirements for each axis shall be as the following clauses.

Orthogonal reference frame

- X-axis

The X axis shall be a straight line passing through the center of the rotating ellipsoid and the intersection of the meridian of zero degrees longitude with the equator.

The value from the center of the rotating ellipsoid to the intersection of the meridian of the zero-degree longitude and the equator shall be positive.

- Y-axis

The Y-axis shall be a line passing through the center of the rotating ellipsoid and the intersection of the meridian of 90 degrees east longitude with the equator.

The value from the center of the rotating ellipsoid to the intersection of the meridian of 90 degrees east longitude and the equator shall be positive.

- Z-axis

The Z-axis shall coincide with the minor axis of the ellipsoid of revolution and shall be positive when it points northward from the center of the ellipsoid of revolution.

Lunar surface reference

Latitude

Latitude shall be the angle between the equatorial plane and a perpendicular line (vertical line) at a given point on the terrestrial surface. The line connecting points of the same latitude is called the latitudinal line, which is a concentric circle parallel to the equator. The latitudinal line at 0° is the equator, which is the basic plane of the spherical coordinate system. The equator divides the earth into the northern and southern hemispheres.

Longitude

Longitude shall be the angle between the meridian drawn from the North Pole to the South Pole through a given Earth surface point and the prime meridian. All meridians are semi-circular, not parallel, and converge at a point at the north and south poles.

Lunar vertical reference

Altitude

Altitude shall be a vertical distance from the lunar reference ellipsoid.

Equipotential surface

At the time of issuing this document, an equipotential surface for the Moon, accessible for the common use of humanity, has not been established. In the future, as liquids such as water are utilized on the lunar surface, the establishment of the Moon's geoid is recognized as a challenge. Upon the establishment of the Moon's geoid, update the definition of height.

Realization

Lunar reference system shall be established by positional reference points. Positional reference points shall be the position of the retroreflectors on the lunar surface in Table 1.  Each reference points also shall have coordinates values: latitude, longitude and altitude in Table 1. These values are based on DE421. The realization shall incorporate the achievements of future science and turn them into better.

- Laser reflector

- Table 1 ― Reference point coordinates[6]

| No. | Mission | X [m] | Y [m] | Z [m] | R [m] | Long. [deg] | Lat. [deg] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A11 | Apollo 11 | 1591747.845 | 691222.345 | 20397.830 | 1735472.732 | 23.4730729 | 0.6734398 |
| A14 | Apollo 14 | 1652818.934 | -520454.721 | -110361.346 | 1736336.135 | -17.4786483 | -3.6441703 |
| A15 | Apollo 15 | 1554937.875 | 98605.140 | 764412.735 | 1735477.340 | 3.6285073 | 26.1333959 |
| L21 | Luna 21 | 1339388.500 | 802310.872 | 755849.325 | 1734639.009 | 30.9221489 | 25.8323070 |

- (informative)

Lunar basic characteristics

The characteristics of the Earth’s Moon are the followings:

- Mean radius			1737.4 km		(0.2727 of Earth’s)

- Eqatroial radius		1738.1 km		(0.2725 of Earth’s)

- Polar radius			1736.0 km		(0.2731 of Earth’s)

- Surface area			3.793 ×107 km2	(0.074 of Earth’s)

- Surface gravity		1.622 m/s2		(1/6 of Earth’s)

- Synodic rotation period	29.530 587 day 	(29 day 12 h 44 min 2.9 s)

- Sidereal rotation period	27.321 661 day

- Surface temperature		Equator	min -230 ℃	mean -20 ℃	max 120 ℃

- 85°N				mean -120℃	max -40 ℃

- Surface pressure		10-7 Pa 			(day)

- 10-10 Pa 		(night)

- Flattening			0.0012

- Escape velocity		2.38 km/s 		(8 600km/h)

- Orbital semi-major axis	384 399 km

- Perigee			362 600 km 		(356 400 – 370 400 km)

- Apogee			405 400 km 		(404 000 – 406 700 km)

- (informative)

Topographic map example

An example of lunar topographic map is given in Figure B.1 and B.2. This topographic map is produced from precise altitude data at a range of up to 5 meters obtained by the laser altimeter (LALT) on board JAXA’s lunar explorer KAGUYA (SELENE) on January 7 to 20, 2008.

- Figure B.1－ Lunar nearside topographic map

- Figure B.2－ Lunar farside topographic map

The contour interval is 1 km and elevation measured from the 1737.4-km radius sphere which centre is the lunar gravity centre. The map is in the stereographic projection and the prime meridian, which longitude is 0 degree, includes the mean sub-Earth point on the Moon.

The farther from the centre, the larger map scale is in the stereographic projection. The boundary is expanded twice from the centre.

The far side of the Moon is covered with many craters of all sizes but has very few ‘mare’ regions that are generally dim, flat, and covered with basaltic materials. In contrast, a relatively large area of the near side is covered with mare regions.

In the southern hemisphere on the far side, there is a huge impact basin named the SPA (South Pole-Aitken) and it is the lowest place in the lunar surface. Many of the mare regions are in a circular or elliptical shape due to the formation history being filled with large amounts of lava in the hollow of the original impact basin. But the SPA, the huge impact basin, is not filled with lava. It is considered that this may be due to the difference in the thickness and/or composition of the lunar crust between the SPA and the near side of the Moon.

The National Astronomical Observatory of Japan (NAOJ) oversees analysis and processing of LALT data and the Geographical Survey Institute of Japan (GSI) oversees production of the map.

- (informative)

Selenodesy short history

During the late 16th to the 17th century, initially with the naked eye and later utilizing telescopes, Gilbert, Harriot, Galilei were drawing the lunar surface. Librations were being recognized.

T. Mayer (1723-1762) determined the lunar longitude and latitude through positional observations of the Manilius Crater. In 1750, a lunar map was created. He also determined the inclination of the Moon's rotational axis and the normal to the plane of the ecliptic to be 1°29' and introduced longitude and latitude with the mean Earth direction as longitude zero. The position of the Manilius Crater was determined at 9°2'N, 14°34'E. Additionally, positions for 23+65 other points were measured. This constituted the world's first geodetic network for celestial bodies other than Earth.

J. H. Lambert (1728-1777) conducted micrometer measurements for 200 points within the field of view of a telescope. Although topographic maps were created using rectified orthographic projection, the accuracy did not match that achieved by Mayer.

J. H. Schröter (1745-1816) created detailed lunar descriptions in "Selenotopographische Fragmente." The positional data was provided by Mayer.

W. G. Lohrmann (1796-1840) measured 79 points on the lunar surface using a filar micrometer and published maps of the Moon in various sizes.

W. Beer (1797-1850) and J. H. Mädler (1794-1874), for the determination of physical libration, measured 106 points with the Filar micrometer. They conducted heliometer observations of the Mösting A Crater from 1834 to 1836, contributing to the refinement of lunar maps by providing detailed information.

The refinement of lunar maps using this method continued until the 1960s. In the latter half of the 19th century, photographic technology was introduced. Lunar mapping through photogrammetry persisted until the 1980s.

From 1969 to the present, LLR was introduced, advancing the precision of lunar ephemerides, and lunar topography measurement has been furthered through image surveying and altimetry from lunar probes.

Bibliography

[1]	ISO/IEC 18026:2009, Information technology — Spatial Reference Model (SRM)

[2]	ISO 19111-1:2019, Geographic information — Referencing by coordinates

[3]	ISO 19161-1:2020, Geographic information — Geodetic references — Part 1: International terrestrial reference system (ITRS)

[4]	Goddard Space Flight Center. A Standardized Lunar Coordinate Systems for the Lunar Reconnaissance Orbiter and Lunar Datasets, LRO Project and LGCWG White Paper Version 5, National Aeronautics and Space Administration, 2008

[5]	W. M. Folkner, J. G. Williams, and D. H. Boggs. The Planetary and Lunar Ephemeris DE 421, JPL Memorandum IOM 343R-08-003, 2008

ftp://ssd.jpl.nasa.gov/pub/eph/planets/ioms/de421iom.pdf

ftp://naif.jpl.nasa.gov/pub/naif/generic_kernels/spk/planets/de421_announcement.pdf

[6]	J. G. Williams, D. H. Boggs, and W. M. Folkner. DE421 Lunar Orbit, Physical Librations, and Surface Coordinates, JPL Memorandum IOM 335-JW, DB, WF-20080314-001, 2008

ftp://ssd.jpl.nasa.gov/pub/eph/planets/ioms/de421_moon_coord_iom.pdf

ftp://naif.jpl.nasa.gov/pub/naif/generic_kernels/spk/planets/de421_lunar_ephemeris_and_orientation.pdf

[7]	Archinal et al., Report of the IAU Working Group on Cartographic Coordinates and Rotational Elements:2015, Article number:22(2018), 2018

[8]	R. S. Park et al., The JPL Planetary and Lunar Ephemerides DE440 and DE411, The Astronomical Journal, American Astronomical Society, 2021

[9]	G. H. Heiken, D. T. Vaniman, and B. M. French, eds, The Lunar Source Book: A User’s Guide to the Moon, Cambridge University Press, 1991

[10]	D. E. Wilhelms, The Geologic History of the Moon, U.S. Geological Survey Professional Paper, no. 1348. Washington, D. C.; U. S. Government Printing Office, 1987

[11] W. D. Compton, Where No Man Has Gone Before: A History of Apollo Lunar Exploration Missions, NASA Special Publication, no. 4214. Washington, D. C.: U. S. Government Printing Office, 1989

[12]	D. E. Bowder and J. K. Hughes, Lunar Orbiter Photographic Atlas of the Moon. NASA Special Publication, no. 206. Washington, D. C.; U. S. Government Printing Office, 1971

[13]	Antonin Rulk. Hamlyn Atlas of the Moon, edited by T. W. Rackham. London: Raul Hamlyn Publishing, 1991

[14]	National Geographic Society. The Earth’s Moon, 2nd ed. Washington, D. C.: Cartographic Division, National Geographic Society, 1976